Evacuation Checklist - Preparing for Flooding

 Shore up property with sand bags.

 You might also need plywood, plastic sheeting and lumber to protect your property

 Prepare for a possible evacuation.

o Ensure your vehicle gas tank is full.

o Pack a grab-and-go bag that includes prescription medicines, if necessary.

o Make accommodation arrangements for all family members and pets.

 Remove high-value household items; move items from basement to upper floor.

 Seal hazardous materials such as petroleum products, paint and insecticides in

plastic garbage bags and move them into safe storage or dispose of them safely.

Electrical appliances

 At first sign of flooding, turn off electrical power and leave it off.

o If main electrical switch is in an already-wet basement, contact an electrician or

BC Hydro for further advice and assistance.

 Water can ruin electrical motors. Move all portable electric heaters, power tools and

electrical appliances to a safe place. Consider having a qualified person remove

motors, controls and switches from furnaces, refrigerators and washers.

 Consider moving all thermally-insulated appliances such as freezers and

refrigerators to upper floors of the house.

Gas-fired appliances

 At first sign of flooding, turn off all gas-fired appliances and put out the pilot light.

Turn off the main gas inlet valve, located at the gas meter.

 Do not remove any gas-fired appliances without ensuring that the main gas inlet

valve has been closed.

 There should be no smoking or open flames in the area.

 For a forced air furnace, move the fan motor and fan to a dry storage place.

 Do not drain the hot water tank; it will serve as ballast to secure the tank in place.

 If gas appliances are removed, make gas outlets tight using a plugged valve, a cap,

or a plug on the piping system.

Plumbing

 Remove any basement toilet bowls. Plug these outlets and all other basement

outlets (floor drains, sinks, laundry drains) to prevent floodwater from entering.

 Use wooden plugs or beanbags and secure outlets with plank braces nailed

securely to floor joists.

Fuel Tanks

 Anchor propane tanks to keep them from moving, even if they are full.

 Turn off the tank valve.

 Disconnect the tubing and seal the end.

 Secure the tank with rope or chain heavy enough to resist the force of floodwaters.